Supermarket Edge AI CCTV System
Site Integration & Infrastructure Deployment Plan

Infrastructure Assessment, Router & PoE Switch Upgrades, UPS Sizing, Camera Specifications, and Optional Accessories

Site Location: Pearcedale, Victoria 3912, Australia (Supermarket Operations) Existing Infrastructure: Configured NAS Storage Server, 32-Channel Dahua NVR/DVR System (192.168.20.160, Gateway 192.168.20.254) Edge AI Appliance Spec: Intel Core i5 (10C/16T) ¦ AMD Radeon RX 9060 XT (16GB) ¦ 16GB DDR5 ¦ 512GB NVMe SSD Key Scope Additions: NAS NFS/SMB Mount, 2.5GbE Multi-VLAN Router, 48-Port PoE+ Switch (384W), 1500VA UPS with USB Auto-Shutdown, 32 Verified 4MP Dahua WizSense IP Cameras, and Server Rack Accessories

# 1. Purpose & Integration Scope

The objective of this deployment plan is to define the exact hardware additions, networking upgrades, power protection, and camera specifications required to integrate the new Edge AI CCTV surveillance and retail analytics platform with the supermarket's existing NAS storage server and store network. This plan guarantees complete network isolation (preventing camera traffic from congesting POS terminals), uninterrupted 24/7 recording during power outages, zero-data-loss database protection, and optimal optical coverage across all 32 store zones.

# 2. Existing NAS Server Integration Plan

The supermarket already possesses an operational Network Attached Storage (NAS) server. Instead of purchasing redundant high-capacity SSDs for the edge PC, the edge appliance will stream all continuous 24/7 video recordings directly to the NAS over the local network:

Storage Protocol: The NAS will expose a high-performance NFS v4 (Linux native) or SMB 3.0 share mounted at /mnt/nas_dvr on the Edge PC.

Bandwidth Ingestion: 32 cameras streaming 1080p H.265 at 6.0 Mbps = 192.0 Mbps (~24.0 MB/s continuous sequential write). This utilizes only 21% of a standard Gigabit NAS connection (or <8% of a 2.5 GbE connection), ensuring zero bottleneck.

Retention Sizing: 24.0 MB/s generates ~2.07 TB/day. For standard supermarket compliance: 7-Day retention requires ~14.5 TB; 14-Day retention requires ~29.0 TB; 30-Day retention requires ~62.0 TB of RAID-5 or RAID-6 storage.

Failover Local Ring Buffer: The Edge PC's local 512GB NVMe SSD maintains a 50 GB emergency buffer. If the NAS undergoes maintenance, reboots, or drops offline, the Edge PC automatically buffers up to 35 minutes of 32-camera video locally and flushes to the NAS upon reconnection.

# 3. Network Router & Managed PoE Switch Upgrade Plan

A typical consumer or standard ISP-provided modem-router cannot handle 32 concurrent camera streams alongside POS billing and customer Wi-Fi. It lacks 802.1Q VLAN network segmentation, sufficient PoE power, and 2.5 Gbps bandwidth, causing multicast flooding and packet drops on checkout registers. The following network infrastructure upgrades are specified:

## 3.1 Commercial Security Router / Gateway

| Model | Ports & Speed | Key Capabilities | Est. Price |
| --- | --- | --- | --- |
| TP-Link Omada ER707-M2 (Primary Choice) | 2× 2.5GbE RJ45 + 4× 1GbE + 1× SFP | Hardware multi-WAN failover (4G/5G backup), multi-VLAN isolation (CCTV vs POS vs Wi-Fi), WireGuard VPN, 500k sessions | $165 – $190 USD |
| Ubiquiti UniFi Cloud Gateway Max (UCG-Max) | 5× 2.5GbE RJ45 ports | Full 2.5 Gbps wire-speed routing with IDS/IPS, integrated UniFi SDN console, advanced traffic filtering | $199 – $220 USD |

## 3.2 High-Density PoE+ Managed Switch (Powering 32 Cameras)

| Model | Port Count & PoE Budget | Bandwidth & Uplinks | Est. Price |
| --- | --- | --- | --- |
| TP-Link Omada TL-SG3452P (Top Recommendation) | 48× Gigabit PoE+ ports (384W total budget) | 4× Gigabit SFP slots. Powers 32 cameras (@ ~6.5W = 208W, 54% budget) + 16 spare ports for Edge PC, NAS, POS, APs | $480 – $520 USD |
| Dual TP-Link TL-SG3428MP (Alternative 2-Switch) | 2× 24× PoE+ ports (384W budget each = 768W) | 4× SFP slots. Distributes load across 2 separate 1U switches for physical redundancy | 2× $285 = $570 USD |

# 4. Power Protection & UPS Sizing Plan (Graceful Auto-Shutdown)

Supermarket power grids often experience voltage sags and momentary blackouts during severe weather or refrigeration compressor cycling. A dedicated Uninterruptible Power Supply (UPS) with USB communication signaling is required to prevent immediate brownout crashes and eliminate SQLite database corruption:

• Edge AI PC (Core i5 + RX 9060 XT 16GB): ~220W – 240W
• 32 PoE Cameras (via PoE Switch): ~190W – 210W (32 × ~6W)
• PoE Switch & Router Logic: ~45W
• Existing NAS Server (4–8 Bays): ~60W – 75W
• Total Operational Power Load: ~515W to 570W

| Model | Capacity & Waveform | Runtime & Features | Est. Price |
| --- | --- | --- | --- |
| APC Smart-UPS SMT1500IC (Recommended Commercial) | 1500VA / 1000W Pure Sine Wave | 18–24 mins runtime at 550W. USB signaling, SmartConnect cloud monitoring, AVR line conditioning | $550 – $620 USD |
| CyberPower PR1500ELCD (Cost-Effective Option) | 1500VA / 1500W Pure Sine Wave | 20–26 mins runtime at 550W. USB + Serial port, GreenPower high-efficiency circuit, LCD diagnostic | $460 – $510 USD |

Automated Linux Shutdown Mechanism: A USB communication cable links the UPS to the Edge PC using the native Linux 'apcupsd' or 'NUT' service. When the battery drops below 15% (after ~15 minutes of outage), an automated script executes:
1. Dispatches high-priority power outage alert to the manager's phone and dashboard.
2. Executes SQLite WAL flush ('PRAGMA wal_checkpoint(TRUNCATE)') to write all pending database transactions to disk.
3. Safely unmounts the NAS NFS volume ('umount -f /mnt/nas_dvr') to prevent remote file handle corruption.
4. Issues clean Linux ACPI 'poweroff' to safely shut down the Edge PC.

# 5. Camera Specifications Checklist (32 Channels)

To ensure 100% optical performance for both human surveillance and AI computer vision (YOLOv8 person tracking, shelf wrist reach, and theft detection), the 32 cameras are mapped to specific lens focal lengths, sensors, and form factors:

| Zone / Dept | Qty | Recommended Model | Optics & Lens | AI & Vision Purpose |
| --- | --- | --- | --- | --- |
| Grocery Aisles 1–12 | 12 | Dahua IPC-HDW2441T-S | 3.6mm Fixed Lens (88° FOV), 4MP Starlight, 120dB WDR, 30m IR | Long corridor perspective, down-aisle tracking, trolley clash, shelf dwell detection |
| Fresh Produce & Bakery | 4 | Dahua IPC-HDW2441T-S | 2.8mm Wide Lens (107° FOV), 4MP Starlight, 120dB WDR | Wide department overview, color fidelity for fresh goods, produce display density |
| POS Checkouts 1–6 | 6 | Dahua IPC-HDW2441T-S | 3.6mm or 6mm Narrow Lens, 4MP, 30 FPS, Built-in Mic | Narrow focus on barcode scanner plate and cash drawer for sweet-hearting & queue wait times |
| Entrances & Foyers | 4 | Dahua IPC-HDBW2441E-S | 2.8mm Wide Lens, 4MP, True WDR (120dB), IK10 Vandal Dome | Counters bright daylight glare from glass doors; customer entry/exit tripwire counts |
| High-Value & Logistics | 6 | Dahua IPC-HDW2441T-S + IPC-HFW2441S-S (Bullet) | 3.6mm Lens, 4MP Starlight, Built-in Mic, IP67 Weatherproof | High-theft shelf reach tracking (Liquor & Pharmacy), loading dock logistics, walk-in cold chain |

# 6. Additional Infrastructure & Optional Items Checklist

The following physical hardware items must be verified on site. If missing, they should be procured to complete the installation:

| Item Description | Specification / Rating | Category | Est. Price | Status |
| --- | --- | --- | --- | --- |
| Server Equipment Cabinet | 9U or 12U Wall-Mount Enclosure (600mm Depth, Glass Door, Lockable) | Mandatory | $140 – $180 | Procure if missing |
| Cat6 Patch Panel (48-Port) | 1U 19" Cat6 110 Punch-Down or Keystone Jack Patch Panel | Mandatory | $35 – $50 | Procure if missing |
| Solid Copper Cat6 Cable | 305m (1000ft) Box UTP Solid Bare Copper 23AWG (Blue/Grey) | Mandatory | $110 – $140 / box | 2 boxes (~600m) |
| Pass-Through RJ45 Plugs | Cat6 Pass-Through RJ45 8P8C Gold-Plated Connectors + Strain Relief Boots (Pack of 100) | Mandatory | $18 – $22 | Procure |
| 1U Horizontal Cable Duct | 19" 1U Finger Duct Cable Management Panel with Cover | Recommended | $15 – $20 | Procure |
| Rackmount 8-Outlet PDU | 1U 19" Horizontal PDU Power Strip with Surge Suppression (10A/16A) | Mandatory | $35 – $45 | Procure |
| Cat6 Molded Patch Leads | 0.5m & 1.0m Snagless Cat6 Patch Cords (Pack of 35 for switch-to-panel) | Mandatory | $28 – $35 | Procure |
| UPS USB Communication Cable | Type-A to Type-B or RJ50-to-USB Cable for APC/CyberPower signaling | Mandatory | $10 – $12 | Included in UPS box |

# 7. Complete Turnkey Procurement Budget

| Procurement Package | Included Components | Subtotal Range (USD) |
| --- | --- | --- |
| 1. Edge AI Appliance PC | Intel Core i5, RX 9060 XT 16GB, 16GB DDR5, 512GB NVMe, 650W PSU, Case | $833 – $953 USD |
| 2. Network Router & PoE Switch | TP-Link ER707-M2 2.5GbE Router + TL-SG3452P 48-Port PoE+ Switch (384W) | $645 – $710 USD |
| 3. Battery Protection (UPS) | APC Smart-UPS 1500VA or CyberPower PR1500ELCD with USB signaling | $460 – $620 USD |
| 4. Rack, Cabling & Accessories | 9U Wall Rack, 48-port Patch Panel, 2x Cat6 boxes, PDU, Patch cords | $391 – $484 USD |
| 5. 32x Cameras (If Procuring New) | 32× Dahua WizSense 4MP WDR Eyeball/Dome IP Cameras (@ ~$75–$85/unit) | $2,400 – $2,720 USD (Optional if existing) |

TURNKEY INFRASTRUCTURE (EXCLUDING CAMERAS IF EXISTING): $2,329 – $2,767 USD
TOTAL COMPLETE SYSTEM (EDGE PC + ROUTER + SWITCH + UPS + RACK + 32 CAMERAS): $4,729 – $5,487 USD