IGA Supermarket — Bill of Materials (BOM)

32-Channel Edge AI Surveillance | Base: Intel Core i5 | RX 9060 XT (16GB) | 16GB DDR5 | 512GB SSD + NAS Storage

Mandatory Materials

| Component | Description / Specification | Qty |
| --- | --- | --- |
| Dedicated AI GPU | AMD Radeon™ RX 9060 XT (16GB GDDR6, 128-bit) [Alt: RX 7600 XT 16GB] | 1 |
| Host Processor (CPU) | Intel Core i5-13400F or i5-14400F (10 Cores, 16 Threads, 65W TDP) | 1 |
| Motherboard | Intel B760M Micro-ATX (2.5GbE LAN, Dual M.2 PCIe 4.0) | 1 |
| System Memory (RAM) | 16GB (2x 8GB) DDR5-5200 MHz CL36 Dual-Channel Kit | 1 |
| Local Storage (SSD) | 512GB M.2 PCIe 4.0 NVMe SSD (WD Blue SN580 / Crucial P3 Plus) | 1 |
| Power Supply (PSU) | 650W 80+ Bronze / Gold Certified ATX Power Supply | 1 |
| Computer Chassis | Micro-ATX High-Airflow Mesh Case with 4x Pre-installed Cooling Fans | 1 |
| CPU Cooler | Thermalright Assassin X 120 Refined SE (120mm Tower Cooler) | 1 |
| Secondary Network Card | PCIe 2.5 GbE Network Card (Realtek RTL8125B) for isolated CCTV VLAN | 1 |
| 48-Port PoE+ Switch | TP-Link Omada TL-SG3452P (48× GbE PoE+, 384W Power Budget, 4× SFP) | 1 |
| Cat6 Patch Panel | 1U 19" 48-Port Cat6 Unshielded Patch Panel (110 Punch-Down / Keystone) | 1 |
| Rackmount 8-Outlet PDU | 1U 19" Horizontal PDU Power Strip with Surge Suppression | 1 |
| Cat6 Molded Patch Leads | 0.5m & 1.0m Snagless Slim Cat6 Patch Cords (Switch to Panel) | 35 |
| RJ45 Connectors | Cat6 Pass-Through RJ45 8P8C Plugs + Strain Relief Boots | 100 pk |
| Existing NAS Storage | Supermarket NAS mount via NFS/SMB for 24/7 video loop (24 MB/s stream) | Existing |

Optional Materials (Site-Dependent / Upgrades)

| Component | Description / Specification | Qty |
| --- | --- | --- |
| Commercial 2.5GbE Router | TP-Link Omada ER707-M2 (2x 2.5GbE, 4x GbE, 802.1Q VLANs) — Needed if current router lacks 2.5GbE/VLANs | 1 |
| Battery Backup (UPS) | APC Smart-UPS SMT1500IC 1500VA/1000W Pure Sine Wave with USB cable — Needed if site lacks 1500VA UPS | 1 |
| Surveillance Cameras (32x) | 32× Dahua 4MP WizSense IP Cameras (Aisles, Produce, Bakery, Checkouts with Mic, IK10 Domes) — If replacing | 32 |
| 32GB RAM Upgrade | 32GB (2x 16GB) DDR5-5600 CL30 — Upgrades base 16GB for 10GB RAM-disk pre-alarm buffer | 1 |
| Equipment Server Cabinet | 9U or 12U Wall-Mount Network Enclosure (600mm Depth, Glass Door) — If rack space unavailable | 1 |
| Bulk Cat6 Network Cable | 305m (1000ft) Box Solid Bare Copper Cat6 UTP 23AWG (2 boxes = 610m) — If re-cabling runs | 2 boxes |
| Horizontal Cable Manager | 1U 19" Finger Duct Cable Management Panel with Removable Cover | 1 |
| Cellular Failover Modem | 4G/5G USB LTE Dongle (Plugs into router for automatic cellular internet failover) | 1 |