Standalone Edge Computing Hardware Comparison

In-Depth Evaluation of Power, Cost, Video Decoding Density, Smooth 24/7 Operation & Feature Implementability for Home CCTV AI

■ EXECUTIVE EVALUATION SUMMARY Choosing an edge computing appliance for in-house CCTV analytics requires balancing three conflicting constraints: Video Decoding Density (hardware VPU/NVDEC), AI Compute (TOPS/VRAM), and Thermal/Power Efficiency. This section provides an empirical benchmark and comparison across all major edge platforms.

1. Comprehensive Edge Device Comparison Matrix

| Edge Hardware Platform | Total System Cost (USD) | Power Draw (Idle / Load) | Hardware Video Decode | AI Compute (TOPS & Stack) | Max Reliable Cameras & Feature Rating |
| --- | --- | --- | --- | --- | --- |
| Intel N100 Mini PC (16GB RAM + 1TB NVMe) | $150 – $190 (Complete PC) | 6W Idle 18W Peak | Intel QuickSync (VA-API) Handles 12-16 sub-streams easily | Intel UHD iGPU OpenVINO (INT8/FP16) ~4-6 TOPS equivalent | 4 – 8 Cameras Motion + Door + Basic Pose. ★★★★☆ (Best Budget Value) |
| Intel N100 / Core i3 + Hailo-8 M.2 Module | $280 – $360 (PC + NPU) | 8W Idle 22W Peak | Intel QuickSync (VA-API) Handles 16-20 sub-streams | Hailo-8 (26 TOPS) HailoRT Dataflow (Zero CPU AI load) | 8 – 16 Cameras Full Fall + Pose + Tracking. ★★★★★ (Top Reliability/Cost) |
| NVIDIA Jetson Orin Nano (8GB Developer Kit / Box) | $350 – $420 (Board + NVMe + Case) | 7W Idle 15W – 25W Peak | GStreamer / DeepStream (No discrete NVDEC; CPU/GPU) | 40 TOPS Ampere GPU + TensorRT (CUDA native) | 6 – 10 Cameras Advanced Pose + Multitask AI. ★★★★★ (Best AI Ecosystem) |
| NVIDIA Jetson Orin NX (16GB Module + Carrier) | $550 – $690 (Industrial Box) | 10W Idle 25W – 40W Peak | Dedicated NVDEC Engine Handles 20+ 1080p streams | 100 TOPS Ampere GPU + TensorRT (Massive inference headroom) | 16 – 20+ Cameras Full Estate Vision AI Suite. ★★★★★ (Enterprise Powerhouse) |
| Rockchip RK3588 (Orange Pi 5 Plus 16GB) | $140 – $180 (SBC + NVMe + PSU) | 4W Idle 14W Peak | Rockchip MPP VPU Multi-channel 4K/8K decode | 6 TOPS NPU RKNN Toolkit (INT8 quantized) | 4 – 8 Cameras Motion + Door + Light Pose. ★★★☆☆ (High setup friction) |
| Raspberry Pi 5 (8GB) + Hailo-8L / AI HAT | $160 – $210 (Pi + HAT + NVMe + Case) | 4W Idle 12W Peak | Software / CPU decode (Decoders limited; CPU bottleneck) | 13 – 26 TOPS HailoRT NPU (Fast inference) | 3 – 5 Cameras Decoding bottleneck limits scaling. ★★★☆☆ (Tinkerer/Small setup) |

2. Detailed Platform Analysis & Operational Feasibility

2.1 Intel N100 / Core i3 Mini PCs (The Operational Stability Winner)

For residential CCTV systems, Intel x86 Mini PCs (N100, N97, Core i3-N305) provide the highest stability and lowest engineering friction.

Hardware Video Decoding Advantage:  Intel QuickSync video hardware decoders process 12 to 16 RTSP H.264/H.265 sub-streams concurrently with < 10% CPU usage.

Zero-Friction Software:  Native x86 Docker compatibility allows standard Home Assistant, Frigate, go2rtc, and Mosquitto containers to run with zero driver compilation or cross-architecture issues.

Expansion Capability:  Adding a 26 TOPS Hailo-8 M.2 module completely offloads YOLOv8 and Pose estimation from the CPU/iGPU, scaling the box to 16 cameras.

2.2 NVIDIA Jetson Orin Nano / NX (The Computer Vision Leader)

If the primary requirement is complex AI pipelines (e.g., continuous 17-keypoint skeleton tracking on multiple people simultaneously, gait anomaly classification, multi-camera ReID):

Unmatched AI Toolchain:  JetPack 6 + TensorRT enables native PyTorch/YOLO conversion with near-zero latency and deep precision (FP16/INT8).

Multitask AI Capacity:  Handles heavy multitask models (object detection + pose + door classification + audio spectrograms) simultaneously in shared unified memory.

Decoding Consideration:  Jetson Orin Nano lacks a dedicated discrete NVDEC chip (decoding uses CPU/GPU GStreamer blocks), meaning sub-stream resolution must be kept at 640x360 @ 5 FPS for > 8 cameras. For 10-20 cameras, the Jetson Orin NX (which includes dedicated NVDEC) is required.

2.3 Rockchip RK3588 (Cost-Effective Embedded Alternative)

The RK3588 SoC (Orange Pi 5 Plus, Radxa Rock 5B) delivers an impressive hardware package ($140 for 16GB RAM, dual 2.5GbE LAN, 6 TOPS NPU).

Strength:  The Rockchip MPP hardware decoder handles multi-channel video efficiently.

Limitation:  The RKNN software stack requires manual model quantization and conversion. Community container support is less mature than x86 or NVIDIA, requiring more maintenance.

2.4 Raspberry Pi 5 + Hailo-8 AI Kit (Good for Tiny Deployments, Decoding-Limited)

While the Hailo-8 NPU provides 13–26 TOPS of fast inference, the Raspberry Pi 5 lacks hardware H.264/H.265 video decoders. Video decoding must be handled in software by the ARM CPU cores.

Decoding Bottleneck:  At 4 cameras, CPU utilization hits 60–80% purely from video decoding, preventing the Hailo NPU from reaching its full potential.

Verdict:  Recommended only for small 2–4 camera apartments where ultra-low power (5W–10W) is the sole objective.

3. Feature Implementation Possibilities by Platform

The table below shows which security and health-monitoring features can run smoothly and concurrently on each edge computing platform:

| Feature | Intel N100 (iGPU) | Intel N100 + Hailo-8 | Jetson Orin Nano (8GB) | Jetson Orin NX (16GB) |
| --- | --- | --- | --- | --- |
| Human Motion & Tracking (YOLOv8n + ByteTrack) | ✔ Yes (Up to 8 cams) | ✔ Yes (Up to 16 cams) | ✔ Yes (Up to 12 cams) | ✔ Yes (Up to 20 cams) |
| Elderly Fall & Posture (YOLOv8-pose + Kinematics) | ✔ Yes (2-3 active people) | ✔ Yes (6-8 active people) | ✔ Yes (8-10 active people) | ✔ Yes (15+ active people) |
| Door State / Left-Open (Polygonal ROI Matcher) | ✔ Yes (Zero overhead) | ✔ Yes (Zero overhead) | ✔ Yes (Zero overhead) | ✔ Yes (Zero overhead) |
| Package Theft / Removal (Stationary Object Tracker) | ✔ Yes | ✔ Yes | ✔ Yes | ✔ Yes |
| Danger Zone Immobility (Bathroom / Inactivity Timers) | ✔ Yes | ✔ Yes | ✔ Yes | ✔ Yes |
| Cross-Camera ReID (Appearance Feature Matching) | ✖ High Latency | ✔ Yes (INT8 Embeddings) | ✔ Yes (TensorRT FP16) | ✔ Yes (Real-Time 20 Cams) |

4. Recommended Decision Blueprint by Residence Size

Small Residence (2 – 4 Cameras):  Intel N100 Mini PC ($160, 15W). Uses Intel QuickSync for video decoding and OpenVINO for YOLOv8 object and basic pose detection. Most cost-effective, plug-and-play x86 appliance.

Medium Home / Villa (5 – 10 Cameras):  NVIDIA Jetson Orin Nano 8GB ($380, 15W) OR Intel N100 + Hailo-8 M.2 ($310, 20W). Delivers robust multi-camera skeletal pose tracking and zero-latency fall classification with zero cloud reliance.

Large Property / Assisted Living (10 – 20 Cameras):  NVIDIA Jetson Orin NX 16GB ($590, 35W) OR Intel Core i5-13500 + Hailo-8 ($550, 45W). Dedicated hardware NVDEC decoding and 100 TOPS AI compute to run full vision intelligence across 20 concurrent camera streams.